INJECTOR ADJUSTING SCREW WEIGHT LOSS ALARM LOG

July 30, 2005 to June 28, 2007 (Severity, Mild direction)

A mild trend of 12 tests triggered a severity alarm. The surveillance panel investigation revealed the screening of injector adjusting screws for tooling marks as the cause of the mild trend. The surveillance panel implemented an industry correction factor of +19.1 mg effective June 28, 2007. Adding this CF to the applicable reference oil tests resulted in the clearing of the industry alarms and the industry plot no longer shows this mild trend.

Updated 7/30/26