L33 (Rust) INDUSTRY ALARM LOG

December 6, 1995 to December 28, 1995 ( Precision )

Five warning alarms and one action alarm during this period.  Appears to be attributed to three tests results of 2.67 /s mild, -2.06 /s severe, and 2.70 /s mild.  There were no identifiable lab, stand or oil problems.  Alarms cleared on next test result.

December 22, 1995  ( Severity )

One warning alarm.  Not an industry alarm event. There were no identifiable lab, stand or oil problems. Alarm cleared on next test result.

January 15, 1996  to February 1, 1996  ( Severity )

Two warning alarms occurred during this period.  Attributed to six consecutive severe of target results with the last two being –2.39  and  –1.58 /s severe. There were no identifiable lab, stand or oil problems. Alarms cleared on next test result.

July 3, 1997  ( Severity )

One warning alarm occrred.  Not an industry alarm event. There were no identifiable lab, stand or oil problems.  Alarm cleared on next test result.

August 28, 1997 to December 12, 1997 ( Severity and Precision )

There were numerous severity and precision alarms triggered during this period. All the severity alarms were in the mild direction and attributed to three labs using more than one axle cover gasket for the test unit assembly. The Precision alarms are attributed to a combination of somewhat severe results associated two labs and the mild results from the three labs using multiple gaskets.

January 13, 1998  to January 15, 1998  ( Precision )

One action alarm and one warning alarm occurred during this period. Appears to be attributed to three consecutive test results of 2.25 /s, -0.75 /s, and 2.25 /s. There were no identifiable lab, stand or oil problems.  Alarms cleared on next test result.

July 7, 1998  ( Severity )

One warning alarm.  Not an industry alarm event. There were no identifiable lab, stand or oil problems.  Alarm cleared on next test result.

August 14, 1998  to August 17, 1998 ( Precision )

Two warning alarms occurred during this period. There were no identifiable lab, stand or oil problems. Alarms cleared on next test result.

October 10, 1998  ( Severity )

One warning alarm.  Not an industry alarm event. There were no identifiable lab, stand or oil problems.  Alarm cleared on next test result.

April 1, 1999 to September 30, 1999 ( Severity and Precision )

There were five precision EWMA warning alarms and ten severity EWMA alarms (eight warning and two action) triggered during this period.  The alarms are related to severe test results from reference oil 123.  Reference oil 121 is on target and reference oil 123 is trending severe.  The surveillance panel has requested a reblend of reference oil 123.